| Assessed By: |  | Kevin Lovett |  |  |  |  |  |  |  |  | Risk Assessment No: |  |  |  |  | RA05 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard Identified: |  | Asbestos Materials |  |  |  |  |  |  |  |  | Date Reviewed: |  |  |  |  | June 2026 |  |
|  |  |  | A |  |  |  |  | B |  |  |  | C |  |  |  | D |  |
|  |  |  | Who’s at Risk? |  |  |  |  | Severity |  |  |  | Likelihood |  |  |  | Risk Rating |  |
|  |  |  | E |  | C | P |  | 3 | 2 | 1 |  | 3 | 2 |  | 1 | #### B x C = D |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  |  | 4 Medium Risk |  |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - LBBD must undertake a comprehensive asbestos survey of all areas where works are to be undertaken and the results of the asbestos survey are to be passed to Seaxe Contract Services Ltd. prior to works being undertaken; - For demolition and refurbishment works, the minimum required survey in accordance with HSE HSG 264 is a Demolition/Refurbishment Survey; - All previous asbestos Management Surveys are to be passed to Seaxe Contract Services Ltd. for review so that we can assess if there will be any risk during the undertaking of our works; - If we are required to undertake our works in an area where licensed asbestos removal has been completed, we will request confirmation by way of the 4 stage clearance certificate, which should be made available from LBBD; - All persons who could potentially disturb asbestos containing materials during the undertaking of their works must first have asbestos awareness training. This is mandatory; - In the event that a material is discovered on site that is believed to be asbestos STOP WORK and tell others working in the area of the suspected danger. Report to the Supervisor or Foreman the area where the material was found. Do not recommence work until you have been authorised to do so; - Report immediately to your Supervisor any accidental damage to asbestos containing materials during our works; - If you are concerned about potential exposure, report this to the Supervisor immediately. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |