| Assessed By: |  | Kevin Lovett |  |  |  |  |  |  |  |  | Risk Assessment No: |  |  |  |  | RA08 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard Identified: |  | Dust |  |  |  |  |  |  |  |  | Date Reviewed: |  |  |  |  | June 2026 |  |
|  |  |  | A |  |  |  |  | B |  |  |  | C |  |  |  | D |  |
|  |  |  | Who’s at Risk? |  |  |  |  | Severity |  |  |  | Likelihood |  |  |  | Risk Rating |  |
|  |  |  | E |  | C | P |  | 3 | 2 | 1 |  | 3 | 2 |  | 1 | #### B x C = D |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  |  | 4 Medium Risk |  |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Ensure that tools are used that incorporate dust extraction and capture e.g. vacuum cleaners with high performance filters (HEPA); - Introduce general ventilation where possible by opening windows and doors when dust is produced. However ensure that the dust does not introduce a hazard to others; - When practical, dusty operations are to be undertaken outside e.g. cutting of worktops, timber etc.; - Lay down dust when possible with the application of water e.g. during sweeping of dust etc.; - Water is to be added to lay down dust but when this is not possible due to completed works, a vacuum cleaner shall be used. RPE shall also be worn; - FFP3 dust masks are to be worn when dust is produced but ensure that all other forms of mechanical and physical methods are used to eliminate dust particularly during sanding-down and cutting; - Dust masks shall be available on site and worn if required. This shall be monitored by the Site Supervisor to ensure compliance; - Operatives shall be trained on the correct fitting and checking procedures of their RPE to ensure that a good seal is maintained during use. For all RPE use refer to manufacturer’s instructions; - Select ‘no added formaldehyde’ board or low emission MDF E1 and E2 classification low level formaldehyde boards in accordance with European standards; - In situations whereby dust may enter the eyes during cutting and drilling etc. safety goggles shall be worn to reduce the risk. Monitored by the Site Supervisor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |