| Assessed By: |  | Kevin Lovett |  |  |  |  |  |  |  |  | Risk Assessment No: |  |  |  |  | RA09 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard Identified: |  | Electric Shock |  |  |  |  |  |  |  |  | Date Reviewed: |  |  |  |  | June 2026 |  |
|  |  |  | A |  |  |  |  | B |  |  |  | C |  |  |  | D |  |
|  |  |  | Who’s at Risk? |  |  |  |  | Severity |  |  |  | Likelihood |  |  |  | Risk Rating |  |
|  |  |  | E |  | C | P |  | 3 | 2 | 1 |  | 3 | 2 |  | 1 | #### B x C = D |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  |  | 4 Medium Risk |  |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Equipment and leads are to be visually inspected before work commences or at the start of each shift; - Do not use damaged work equipment. Remove it immediately and notify the Supervisor / Foreman; - Do not use the equipment until it has been repaired by a trained electrician and your Supervisor has instructed that it is safe to do so; - Tools and equipment will be Portable Appliance Tested (PAT) every three months and will be 110v Centre tapped to earth or will be low voltage battery operated where possible; - 110v cables are to be routed above head height to reduce the risk of damage occurring to the cables; - Operatives will not be permitted to undertake makeshift repairs e.g. tape jointing of power cables; - Any services that may be affected will be isolated by a trained and competent electrician prior to the works commencing; - Temporary services will be installed and maintained by a competent electrician; - All operatives will be trained, instructed and supervised in the correct use of power tools; - Electrical equipment must not be used in wet conditions; - The electric cupboard / switch room must remain padlocked shut at all times unless an electrician is present. Report immediately if you see a cupboard unattended and left open; - Hazard warning signs will be displayed where electrical hazards still remain and these areas will be suitably controlled. Live headers may require boxing-in and suitable signage displayed; - Any damage to services must be reported immediately; - Carry out additional checks with live cable detectors if for any reason you suspect something is still live; - Report to the Supervisor / Foreman if further services are discovered during the works that have not been previously identified; - Only trained and competent electricians will work on services and services will be isolated before being worked on. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |