| Assessed By: |  | Kevin Lovett |  |  |  |  |  |  |  |  | Risk Assessment No: |  |  |  |  | RA13 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hazard Identified: |  | Muscular Skeletal Disorders |  |  |  |  |  |  |  |  | Date Reviewed: |  |  |  |  | June 2026 |  |
|  |  |  | A |  |  |  |  | B |  |  |  | C |  |  |  | D |  |
|  |  |  | Who’s at Risk? |  |  |  |  | Severity |  |  |  | Likelihood |  |  |  | Risk Rating |  |
|  |  |  | E |  | C | P |  | 3 | 2 | 1 |  | 3 | 2 |  | 1 | #### B x C = D |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |
| Risk Assessment Ratings | 6-9 High Risk |  |  |  |  |  | 4 Medium Risk |  |  |  |  |  |  | 1-3 Low Risk |  |  |  |
| Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A E = Employees C = Contractors P = Public / 3rd parties |  |  |  | B 3 = Death or Major injury 2 = Reportable injury 1 = Minor Injury – Time off unlikely |  |  |  |  |  |  |  | C 3 = Very likely 2 = Possible 1 = Unlikely or Very Unlikely |  |  |  |  |  |
| Hazard Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residual Risk |
| - Mechanical means e.g. sack barrow, trolley, pallet forks shall be used if there is a foreseeable risk of injury due to manual handling; - Where hazardous manual handling activities cannot be avoided, the risk of injury shall be reduced as far as is reasonably practicable by: - Trained operatives using correct manual handling techniques; - Reducing loads and separating into smaller loads; - Training shall be provided in the correct methods of lifting; - Operatives may be supervised to ensure that the best available technique of lifting is being used; - Operatives are never to attempt to lift something that is too heavy for their own capabilities; - Ask for assistance or ensure that a number of persons help with a task if the load cannot be broken down to reduce the weight; - Where two-man lifting is adopted then one is to take control and give directions in lifting co-ordination; - Ensure the route is cleared of all trip hazards, obstructions and persons; - Check for sharp edges and cover where possible; - Repetitive bending, twisting and heavy lifting shall be avoided; - Where possible working in poor conditions shall be avoided e.g. too hot, too cold, etc.; - Operatives are encouraged to report symptoms to their supervisor / manager immediately. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Low (3) |